# The AVM Network Application Programmer's Interface (AVM NW API) Specification

- 11.6.1999 Version 1.63

History

27.2.98 (Version 1.0):

- AVMNW_Connection_Authenticate now handled through the password provider.

- Changed struct AVMNWPasswordProviderParameters for better support of inbound/outbound authentications.

- AVMNW_PasswordProvider_Callback can't detect timeout condition.

- Notification of password provider either by semaphore or Windows message, but not both.

- First implementation for NT/MPRI and ISDN Access Server available.

5.3.98 (Version 1.1):

- Parameters for password provider changed. Usernames and passwords now handled independly.

- Various target attributes added (short hold, masquerading, CLI security...).

- Product object added.

17.3.98 (Version 1.2)

- Parameters for password provider changed. Controller available for username providing.

3.4.98 (Version 1.3)

- Parameter eProdcutModulesRoutingRuntimeAndAdmin added.

- Functions AVMNW_Config_EnableAccessServer and AVMNW_Config_Lock_MPRI added.

29.5.98 (Version 1.4)

- Functions AVMNW_Config_Target_FindFirst/FindNext for enumeration of target objects added.

9.6.98 (Version 1.5)

- Target attribute nActiveMode added (NetWAYS/ISDN only).

27.7.98 (Version 1.51)

- Target attribute nUseIPGateway added (NetWAYS/ISDN only).

- Controller attribute bIgnoreIncomingCalls added.

9.9.98 (Version 1.52)

- IP WAN Route Objects added.

- New concepts added to support IP WAN Route Objects. The concepts will substitute the object type specific function (e.g. AVMNW_Config_Target_Create) in future versions of the API.

- Introduction of Object Types and generic creation and enumeration functions.

- Introduction of an object ID (AVMNW_Attrib_nID) to uniquely identify an object within its type.

- New Target attributes AVMNW_Attrib_szName and AVMNW_Attrib_nID

- Controller attribute bIgnoreIncomingCalls: Restriction to only one controller removed.

- IP-Masquerading supported for NetWAYS/ISDN.

17.9.98 (Version 1.53)

- Corrected type for bUsernameQuery in password provider (BOOL instead of bool).

7.10.98 (Version 1.6)

- All generic AVMNW_Config_Object... functions are supported by all object types.

- nID attribute for all objects added.

- IPXWANRoute and SAPService objects addded.

- IPXSAPFilterSet and IPXSAPFilterRule added.

- Function AVMNW_Config_Object_UseByText added.

- Some new attributes for the Controller added (NetWAYS/ISDN only).

11.2.99 (Version 1.61)

- Added attributes for Product object: Access to budgets, WAN-WAN-routing can be disabled.

- Added attributes for Target object: Access to SNMP filter and data encryption.

24.2.99 (Version 1.62)

- Added attributes for Controller object: Access to budgets (NetWAYS/ISDN).

- Added target attribute bRemoteNode.

- Changed restrictions for access to configuration when the product is running.

11.6.99 (Version 1.63)

- Corrected notes about support for NetWAYS/ISDN.

- Function AVMNW_Connection_InvalidateSessionKey() added.

- Added values for target attribute nDataEncryption to set the size of the session key.

- Added target attributes szCountryCode, szAreaCode and szProviderCode (for NetWAYS/ISDN).

General

The AVM NW API is provided to control various AVM ISDN networking products by third party applications. The API provides functions to do a couple of things from a third party application:

- Configuration

- Product start/stop control,

- Password providing, and

- Connection control.

## Terms

Product

The term product is a placeholder that stands for the specific AVM product that is controlled by this API. The supported products are

- NetWAYS/ISDN for Windows 95,

- NetWAYS/ISDN for Windows NT,

- NT/MPRI (ISDN Multiprotocol Router for Windows NT) and

- ISDN Access Server for Windows NT.

Note that some of the API functions will fail for an ISDN Access server without an installed NT/MPRI. It the ISDN Access Server and the NT/MPRI are installed both, both products are controlled and the API will form the union of the APIs available for the single products.

The actual product is determined by the DLL loaded at runtime. This DLL is called AVMNWAPI.DLL for all products. It knows which actual product it supports (either by hard coding or by decision at runtime).

The use of the API requires a fully installed product (e.g. rebooting after installation must have been done if required).

Product running

An AVM product controlled by this API can be in the running or in the not running state. The definition of this state depends on the actual product.

API DLL

Any process that uses this API has to load a product specific DLL where the API functions are implemented. This DLL is shipped with the product and is named AVMNWAPI.DLL.

The usage of the API is supported by a C++ header file avmnwapi.h and an import library for MS Visual C++. The header file is part of the specification, since it defines the values of constants used.

Configuration Database

The configuration of the product is stored in the configuration database. The database consists of sets of configuration objects (called objects).

Object

An object is part of the configuration database. Every object has a type (e.g. target or controller). Within its type every single object can be uniquely identified by a key attribute (e.g. name or CAPI2.0 controller number).

Attribute

Every object type defines a set of attributes that can be configured for this object. Each attribute has a fixed datatype (e.g. Text or IPRoute).

Current object

The configuration part of the API maintains a focus on a specific object that is called the current object. Only attributes of the current object can be accessed. There can be only one current object at a time. The current object can be undefined also.

## Function Overview and Assigned ordinal numbers

An ordinal number is assigned to every function of this API. This numbers can be used to import the functions of the API DLL.

General

| AVMNW_Register | @100 |
| --- | --- |
| AVMNW_Unregister | @101 |

Configuration

| AVMNW_Config_Open | @110 |
| --- | --- |
| AVMNW_Config_Close | @111 |
| AVMNW_Config_Lock | @112 |
| AVMNW_Config_Lock_MPRI | @113 |
| AVMNW_Config_EnableAccessServer | @114 |
| AVMNW_Config_Object_Unuse | @120 |
| AVMNW_Config_Object_Delete | @121 |
| AVMNW_Config_Object_UseNew | @122 |
| AVMNW_Config_Object_Create | @123 |
| AVMNW_Config_Object_UseByText | @124 |
| AVMNW_Config_Object_UseByNumber | @125 |
| AVMNW_Config_Object_FindFirst | @126 |
| AVMNW_Config_Object_FindNext | @127 |
| AVMNW_Config_Attribute_GetText | @130 |
| AVMNW_Config_Attribute_SetText | @131 |
| AVMNW_Config_Attribute_GetNumber | @132 |
| AVMNW_Config_Attribute_SetNumber | @133 |
| AVMNW_Config_Attribute_GetBool | @134 |
| AVMNW_Config_Attribute_SetBool | @135 |
| AVMNW_Config_Attribute_SetIPRoute | @136 |
| AVMNW_Config_Attribute_GetIPRoute | @137 |

Object type specific – These functions are still supported, but shouldn’t be used any more.

| AVMNW_Config_Target_Create | @150 |
| --- | --- |
| AVMNW_Config_Target_Use | @151 |
| AVMNW_Config_Target_FindFirst | @152 |
| AVMNW_Config_Target_FindNext | @153 |
| AVMNW_Config_Controller_Create | @180 |
| AVMNW_Config_Controller_Use | @181 |
| AVMNW_Config_Product_Use | @185 |

Module Control

| AVMNW_Module_Start | @300 |
| --- | --- |
| AVMNW_Module_Stop | @301 |
| AVMNW_Module_GetStatus | @302 |

Password Providing

| AVMNW_PasswordProvider_Register | @320 |
| --- | --- |
| AVMNW_PasswordProvider_Callback | @321 |

Connection Control

| AVMNW_Connection_Authenticate | @330 |
| --- | --- |
| AVMNW_Connection_InvalidateSessionKey | @331 |

## Functions

AVMNW_RESULT PASCAL AVMNW_Register(void *pReserved);

Parameters

| pReserved | This parameter is reserved and should be set to NULL. |
| --- | --- |

Remarks

This function have to be called by a process before all other functions of this API are allowed to be called.

More than one process may be registered at a time.

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

Return Value

|  | AVMNW_OK |  | Successful registered. |  |
| --- | --- | --- | --- | --- |
|  | AVMNW_ERROR_FAILED |  | Registering failed. |  |
| AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |  |  |
|  | AVMNW_ERROR |  | General error |  |

See Also

AVMNW_Unregister

AVMNW_RESULT PASCAL AVMNW_Unregister(void);

Parameters

There are no parameters.

Remarks

This function have to be called by a process that called AVMNW_Register before the process terminates or unloads the API DLL. This is required to do cleanup tasks.

Return Value

| AVMNW_OK | Successfully unregistered |
| --- | --- |
| AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

See Also

AVMNW_Register

AVMNW_RESULT PASCAL AVMNW_Config_Open(void);

Parameters

There is no parameter.

Remarks

This function have to be called by a process before any of the configuration functions are allowed to be called.

Only one process is allowed to open the configuration at a time.

When the product is running, access to the configuration is possible, but restricted. The remarks to the AVMNW_Config... function define these restrictions. Currently full read access is supported, but only existing target objects can be modified. These restrictions are to be interpreted as ‘something that is not restricted is guaranteed to work, but if it is restricted it may work also’. The API does no formal checks on restrictions when the product is running.

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

Return Value

| AVMNW_OK | The configuration was successfully opened |
| --- | --- |
| AVMNW_ERROR_BUSY | Another process opened the configuration already |
| AVMNW_ERROR_DB | Access to the configuration database failed. |
| AVMNW_ERROR | General error (e.g. the process is not registered) |

See Also

AVMNW_Config_Close

AVMNW_RESULT PASCAL AVMNW_Config_Close(void);

Parameters

There is no parameter.

Remarks

This function have to be called by a process to end any configuration operations.

When returned with AVMNW_OK, the configuration database can be opened again.

If there were any changes to the configuration while the product was running, it is required to restart the product.

NetWAYS/ISDN specific:

Changes to target objects do not require a restart of the product if it is running.  The next time a connection is established to the target the new parameters are used.

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

Return Value

|  | AVMNW_OK |  | The configuration database has been successfully closed. |  |
| --- | --- | --- | --- | --- |
|  | AVMNW_ERROR |  | The process did not successfully call AVMNW_Config_Open before. |  |
|  | AVMNW_ERROR_DB |  | Access to the configuration database failed. |  |
| AVMNW_ERROR |  | General error (e.g. not registered). |  |  |

See Also

AVMNW_Config_Open

AVMNW_RESULT PASCAL AVMNW_Config_Lock(BOOL bLock);

Parameters

| bLock | Lock flag to be set. |
| --- | --- |

Remarks

If bLock is TRUE the user of the product can’t modify the configuration of the product any longer.

If bLock is FALSE the user is enabled to modify the configuration of the product.

Modification is affected concerning the normal user product tools and user interfaces only. Modification of configuration is always allowed when this API is used.

It is not required to open the configuration database to use this function.

It is not allowed to call this function when the product is currently running.

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

Return Value

|  | AVMNW_OK | The function succeeded. |  |
| --- | --- | --- | --- |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |  |

See Also

AVMNW_Config_Lock_MPRI

AVMNW_Config_EnableAccessServer

AVMNW_RESULT PASCAL AVMNW_Config_Lock_MPRI (BOOL bLock);

Parameters

| bLock | Lock flag to be set. |
| --- | --- |

Remarks

If bLock is TRUE the user of the product can’t modify the configuration of the NT/MPRI product any longer.

If bLock is FALSE the user is enabled to modify the configuration of the NT/MPRI product.

Modification is affected concerning the normal user product tools and user interfaces only. Modification of configuration is always allowed when this API is used.

It is not required to open the configuration database to use this function.

It is not allowed to call this function when the product is currently running.

The general lock flag set by AVMNW_Config_Lock superseeds this function, so the general lock flag should be FALSE to see any effect from this function.

It makes sense to use this function only when the NT/MPRI and the ISDN Access Server are both installed.

Product Specifics

NetWAYS for Windows 95 – not supported

NetWAYS for Windows NT –not  supported

NT/MPRI – supported, see above

ISDN Access Server – not supported, see above

Return Value

|  | AVMNW_OK | The function succeeded. |
| --- | --- | --- |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR_NOTSUPPORTED | This function is not supported (e.g. the ISDN Access Server is not installed) |
|  | AVMNW_ERROR | General error (e.g. not registered). |

See Also

AVMNW_Config_Lock

AVMNW_Config_EnableAccessServer

AVMNW_RESULT PASCAL AVMNW_Config_EnableAccessServer(BOOL bEnable);

Parameters

| bEnable | Flag for enabling of the ISDN Access Server. |
| --- | --- |

Remarks

If bEnable is TRUE product ISDN Access Server will be enabled.

If bEnable is FALSE the product ISDN Access Server will be disabled. The next time the product is started, the functionality of the ISDN Access (remote nodes) will not be available.

The ISDN Access Server have to be already installed (including a valid CD key).

It is not required to open the configuration database to use this function

It is not allowed to call this function when the product is currently running.

Product Specifics

NetWAYS for Windows 95 – not supported

NetWAYS for Windows NT – not supported

NT/MPRI – not supported, see above

ISDN Access Server – supported, see above

Return Value

|  | AVMNW_OK | The function succeeded. |
| --- | --- | --- |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR_NOTSUPPORTED | This function is not supported (e.g. the ISDN Access Server is not installed) |
|  | AVMNW_ERROR | General error (e.g. not registered). |

See Also

AVMNW_Config_Lock

AVMNW_Config_Lock_MPRI

AVMNWFUNC AVMNW_Config_Object_UseNew(AVMWN_ObjectType OjectType);

Parameters

| ObjectType | The type of object to be created. |
| --- | --- |

Remarks

A new object of the specified object type ObjectType will be created. The object will become the current object. All modified attribute values in the current object are silently discarded.

When this function returns with AVMNW_OK, the attribute values of the object can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

The new object is created in a temporary pre-creation state. All attribute values are set to default values.

The pre-creation state has the following characteristics:

- The object has no valid value of the AVMNW_Attrib_nID attribute.

- Conflicts of attribute values with other objects may exist (e.g. non unique target names).

- It can not be enumerated with ..FindFirst  ...FindNext.

The promote an object from the pre-creation to a persistent state, the function AVMNW_Config_Object_Create or AVMNW_Config_Object_Unuse with TRUE should be used.

If the current object is changed (without calling AVMNW_Config_Object_Create) or by calling AVMNW_Config_Object_Unuse with FALSE the temporary object will be automatically deleted.

It is not allowed to call this function when the product is currently running.

Currently only the Object Type IPWANRoute is supported.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTSUPPORTED | The object type is not supported by the product or object creation of this type is not allowed (e.g. product object). |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the object. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 –supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server - supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_Unuse

AVMNWFUNC AVMNW_Config_Object_Create(/*OUT*/unsigned int *pID);

Parameters

| pID | A memory location that will receive the ID assigned to the new persistent object. |
| --- | --- |

Remarks

The current object is promoted from the pre-creation the persistent state. Attribute values are checked for confilcts with other objects of the same type and references to other objects are checked.

Finally a unique ID is assigned to the object and stored in AVMNW_Attrib_nID. The ID is returned in *pID.

The current object will be undefined. If you want to use the object again, you can call AVMNW_Config_Object_UseByNumber(ObjectType, AVMNW_Attrib_nID, nID), where ObjectType is the type of the object and nID is the ID retured in *pID.

It is not allowed to call this function when the product is currently running.

Currently only the Object Type IPWANRoute is supported.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTSUPPORTED | The object type is not supported by the product or object creation of this type is not allowed (e.g. product object). |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the object. For example this may happen if the product is currently running. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 - supported

NetWAYS for Windows NT - supported

NT/MPRI - supported

ISDN Access Server - supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_UseNew

AVMNW_Config_Object_Unuse

AVMNWFUNC AVMNW_Config_Object_FindFirst(AVMWN_ObjectType OjectType , /*OUT*/unsigned int *pID);

Parameters

| ObjectType | The type of object to be found. |
| --- | --- |
| pID | A memory location that will receive the ID of the object found. |

Remarks

The function delivers the ID of the first object of the specified ObjectType. Objects are not sorted.

The API keeps an internal enumeration information that is affected by AVMNW_Config_Object_FindFirst and AVMNW_Config_Object_FindNext. The information is kept per object type, so FindFirst/FindNext can be nested for different object types.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The ID of the first object of the given type has been delivered. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | There is no object of this type. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server - supported

See Also

AVMNW_Config_Object_FindNext

AVMNWFUNC AVMNW_Config_Object_FindNext(AVMWN_ObjectType OjectType, /*OUT*/unsigned int *pID);

Parameters

| ObjectType | The type of object to be found. |
| --- | --- |
| pID | A memory location that will receive the ID of the object found. |

Remarks

The function delivers the ID of the next object of the specified ObjectType. Objects are not sortet.

The API keeps an internal enumeration information that is affected by AVMNW_Config_Object_FindFirst and AVMNW_Config_Object_FindNext. The information is kept per object type, so FindFirst/FindNext can be nested for different object types.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The ID of the next object of the given type has been delivered. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | There is no more object of the given type. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server - supported

See Also

AVMNW_Config_Target_FindFirst

AVMNWFUNC AVMNW_Config_Object_UseByText(AVMNW_ObjectType ObjectType, AVMNW_Attrib TextAttribute, char *szTextValue);

Parameters

| ObjectType | The type of object to become the current object. |
| --- | --- |
| TextAttribute | The attribute of type text that is used to identify the object. |
| szTextValue | The text value that is used to identify the object. |

Remarks

The object of the specified ObjectType which have the attribute TextAttribute set to the value szTextValue will become the current object. All modified attribute values in the current object are silently discarded.

Only attributes of type text that are marked with “Objects can be ‘used’ by this attribute” in the object schema of the product can passed in TextAttribute.

When this function returns with AVMNW_OK, the attribute values of the object can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The object was not found. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_UseByNumber

AVMNW_Config_Object_Unuse

AVMNWFUNC AVMNW_Config_Object_UseByNumber(AVMNW_ObjectType ObjectType, AVMNW_Attrib NumberAttribute, unsinged int nNumberValue);

Parameters

| ObjectType | The type of object to become the current object. |
| --- | --- |
| NumberAttribute | The attribute of type number that is used to identify the object. |
| nNumberValue | The number value that is used to identify the object. |

Remarks

The object of the specified ObjectType which have the attribute NumberAttribute set to the value nNumberValue will become the current object. All modified attribute values in the current object are silently discarded.

Only attributes of type Number that are marked with “Objects can be ‘used’ by this attribute” in the configuration schema of the product can passed in NumberAttribute.

When this function returns with AVMNW_OK, the attribute values of the object can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The object was not found. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_Unuse

AVMNW_RESULT PASCAL AVMNW_Config_Object_Delete(void);

Parameters

There are no parameters.

Remarks

Use this function to delete the current object.

If AVMNW_OK is returned, the current object will be undefined.

It is not allowed to call this function when the product is currently running.

Return Value

| AVMNW_OK | The object have been deleted. |
| --- | --- |
| AVMNW_ERROR_NOTFOUND | The current object is not defined. |
| AVMNW_ERROR_DB | Failure to access the configuration database. |
| AVMNW_ERROR_NOTSUPPORTED | Deletion of this object is not supported by the controlled product. (e.g. NetWAYS/ISDN does not supported deletion of controller objects) |
| AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

See Also

AVMNW_Config_Target_Create

AVMNW_Config_Controller_Create

AVMNW_RESULT PASCAL AVMNW_Config_Object_Unuse(
   BOOL bCommitChanges, /*OUT*/AVMNW_Attrib *pAttribute);

Parameters

| bCommitChanges | Defines if changed attributes of the current object should be permanently stored or discarded. Use TRUE to keep changes and FALSE to discard them. |
| --- | --- |
| pAttribute | If AVMNW_ERROR_CONFIG is returned, pAttribute will receive the attribute (id) involved in the illegal configuration. |

Remarks

The attribute values of the current object are checked for an illegal value combination.

If such an illegal configuration is found, the id of an involved attribute is stored in *pAttribute and AVMNW_ERROR_CONFIG is returned. The current object is not modified.

If the configuration is ok and bCommitChanges is TRUE, all changes to attribute values are stored in the database.

The current object will be undefined (except when AVMNW_ERROR_CONFIG) is returned.

When the product is running, bCommitChanges can be TRUE for target objects only. That means no other objects support configuration changes while the product is running.

Return Value

|  | AVMNW_OK | The modifications to the current object were saved to the database or were discarded. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined or was not found in the database. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to write the object. |
|  | AVMNW_ERROR_CONFIG | There is an illegal combination of two or more attributes of the current object. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

See Also

AVMNW_Config_..._Use

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_GetText(
   AVMNW_Attrib Attribute,
   /*OUT*/char szText,
   /*INOUT*/unsigned int *pnMaxSize);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| szText | A character buffer that will receive the ‘\0’ terminated text value. |
| pnMaxSize | A memory location that contains the size of szText in byte. |

Remarks

This function gets the value of a text attribute of the current object.

On input, the parameter *pnMaxSize should contain the size of the szText buffer (number of characters including a terminating ‘\0’ byte).

On return of AVMNW_OK, *pnMaxSize will contain the length of the value (including terminating ‘\0’ byte). So if it is greater than set on input, the value has been truncated.

There are no restrictions when the product is currently running.

Return Value

|  |  | AVMNW_OK | The attribute has been retreived successfully. |
| --- | --- | --- | --- |
|  |  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  |  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a text attribute. |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR |  | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_SetText

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_SetText(
   AVMNW_Attrib Attribute,
   char *szText);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| szText | A character buffer containing the ‘\0’ terminated text value. |

Remarks

This function modifies the value of a text attribute of the current object.

AVMNW_Config_Object_Unuse has to be called to permanently store the attribute value in the configuration database.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute have been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a text attribute. |
|  | AVMNW_ERROR_CONFIG | The given value is not allowed for this attribute (e.g. too many chars). |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_GetText

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_GetNumber(
   AVMNW_Attrib Attribute,
   /*OUT*/ unsigned int *pNumber);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| pNumber | A memory location that will receive the attribute value. |

Remarks

This function gets the value of a number attribute of the current object.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute value have been retrieved successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a number attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_SetNumber

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_SetNumber(AVMNW_Attrib Attribute, unsigned int Number);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| Number | The value to be assigned to the attribute. |

Remarks

This function modifies the value of a number attribute of the current object.

AVMNW_Config_Object_Unuse has to be called to permanently stored the attribute value in the configuration database.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute value has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a number attribute. |
|  | AVMNW_ERROR_CONFIG | The given value is not allowed for this attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_GetNumber

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_GetBool(
   AVMNW_Attrib Attribute,
   /*OUT*/ BOOL *pBool);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| pBool | A memory location that will receive the attribute value. |

Remarks

This function gets the value of a boolean attribute of the current object.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute has been retrieved successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a boolean attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_SetBool

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_SetBool(AVMNW_Attrib Attribute, BOOL Bool);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| Bool | The value to be assigned to the attribute. |

Remarks

This function modifies the value of a boolean attribute of the current object.

AVMNW_Config_Object_Unuse has to be called to permanently store the attribute value in the configuration database.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not a boolean attribute. |
|  | AVMNW_ERROR_CONFIG | The given value is not allowed for this attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_GetBool

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_GetIPRoute(
   AVMNW_Attrib Attribute,
   /*OUT*/AVMNW_IPRoute *pIPRoute);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| pIPRoute | A memory location that will receive the IPRoute attribute value. |

Remarks

This function gets the value of an IPRoute attribute of the current object.

pIPRoute->nSize have to be set to sizeof(AVMNW_IPRoute) on input.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not an IPRoute attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_SetIPRoute

AVMNW_RESULT PASCAL AVMNW_Config_Attribute_SetIPRoute(
   AVMNW_Attrib Attribute,
   AVMNW_IPRoute *pIPRoute);

Parameters

| Attribute | Specifies the attribute. This can be any of the AVMNW_Attrib_... values that are defined for the current object. |
| --- | --- |
| pIPRoute | The value to be assigned to the attribute. |

Remarks

This function modifies the value of an IPRoute attribute of the current object.

pIPRoute->nSize have to be set to sizeof(AVMNW_IPRoute) on input.

AVMNW_Config_Object_Unuse has to be called to permanently store the attribute value in the configuration database.

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The attribute has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The current object is not defined. |
|  | AVMNW_ERROR_ATTRIB | The specified attribute is not defined for the current object or is not an IPRoute attribute. |
|  | AVMNW_ERROR_CONFIG | The given value is not allowed for this attribute. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support this attribute |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_GetIPRoute

AVMNW_RESULT PASCAL AVMNW_Config_Target_Create(char *szName); // OUTDATED

Parameters

| szName | The name of the target to be created. |
| --- | --- |

Remarks

Use this function to create a new target. A new PPP target with the given name szName and default attributes will be created.

It is not allowed to call this function when the product is currently running.

ISDN Access Server specific:

The target created will be a LAN target and no user, so the NT/MPRI has to be installed too.

Return Value

|  | AVMNW_OK | The new target has been created. |
| --- | --- | --- |
|  | AVMNW_ERROR_EXIST | A target with the given name already exists. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to create the new target. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support target objects (e.g. ISDN Access Server without NT/MPRI). |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- not supported, see above

See Also

AVMNW_RESULT PASCAL AVMNW_Config_Target_Use(char *szName); // OUTDATED

Parameters

| szName | The name of the target to be used. |
| --- | --- |

Remarks

The target object with the specified name szName will become the current object. All modified attribute values in the current object are silently discarded.

When this function returns with AVMNW_OK, the attribute values of the target can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

There are no restrictions when the product is currently running.

ISDN Access Server specific:

This function will also allow access to internally administrated users since they share the same name space with LAN targets in the NT/MPRI.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The target was not found. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_Unuse

AVMNW_RESULT PASCAL AVMNW_Config_Target_FindFirst(/*OUT*/char szName, /*INOUT*/unsigned int *pnMaxSize); // OUTDATED

Parameters

| szName | A character buffer that will receive the ‘\0’ terminated name of the first target. |
| --- | --- |
| pnMaxSize | A memory location that contains the size of szName in byte. |

Remarks

The function delivers the name the first target object. Objects are not sortet. The parameters are handled like in AVMNW_Config_Attribute_GetText().

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The name of the first target object has been delivered. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | There is no target object. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Target_FindNext

AVMNW_Config_Attribute_GetText

AVMNW_RESULT PASCAL AVMNW_Config_Target_FindNext(/*OUT*/char szName, /*INOUT*/unsigned int *pnMaxSize); // OUTDATED

Parameters

| szName | A character buffer that will receive the ‘\0’ terminated name of the next target. |
| --- | --- |
| pnMaxSize | A memory location that contains the size of szName in byte. |

Remarks

The function delivers the name the next target object. Objects are not sortet. The parameters are handled like in AVMNW_Config_Attribute_GetText().

The actual target object is determined by an API internal target enumeration information that is affected by the AVMNW_Config_Target_FindFirst() and AVMNW_Config_Target_FindNext().

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The name of the next target object has been delivered. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | There is no more target object. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Target_FindFirst

AVMNW_Config_Attribute_GetText

AVMNW_RESULT PASCAL AVMNW_Config_Controller_Create(unsigned int CAPI20Controller); // OUTDATED

Parameters

| CAPI20Controller | The CAPI 2.0 Controller Number of the controller to be created. |
| --- | --- |

Remarks

Use this function to create a new controller object in the configuration database.

It is not allowed to call this function when the product is currently running.

Return Value

|  | AVMNW_OK | The new controller has been created. |
| --- | --- | --- |
|  | AVMNW_ERROR_EXIST | A controller with the given controller number already exists. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to create the new target. |
|  | AVMNW_ERROR_NOTSUPPORTED | The controlled product does not support controller creation (e.g. NetWAYS/ISDN). |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – not supported, see above

NetWAYS for Windows NT – not supported, see above

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_RESULT PASCAL AVMNW_Config_Controller_Use(unsigned int CAPI20Controller); // OUTDATED

Parameters

| CAPI20Controller | The CAPI 2.0 Controller Number of the controller to be used. |
| --- | --- |

Remarks

The controller object with the specified controller number CAPI20Controller will become the current object. All modified attribute values in the current object are silently discarded.

When this function returns with AVMNW_OK, the attribute values of the controller can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The controller was not found. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_Unuse

AVMNW_RESULT PASCAL AVMNW_Config_Product_Use(void); // OUTDATED

Parameters

There are no parameters.

Remarks

The product object will become the current object. All modified attribute values in the current object are silently discarded.

When this function returns with AVMNW_OK, the attribute values of the product can be accessed by AVMNW_Config_Attribute_Get... and AVMNW_Config_Attribute_Set....

There are no restrictions when the product is currently running.

Return Value

|  | AVMNW_OK | The current object has been set successfully. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTSUPPORTED | The product object is not supported. |
|  | AVMNW_ERROR_DB | Failure to access the configuration database or to read the target. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – not supported

NetWAYS for Windows NT – not supported

NT/MPRI – supported

ISDN Access Server- supported

See Also

AVMNW_Config_Attribute_Get...

AVMNW_Config_Attribute_Set...

AVMNW_Config_Object_Unuse

AVMNW_RESULT PASCAL AVMNW_Module_Start(
   enum AVMNWProductModules eProductModules,
   /*OUT*/HANDLE *pHandle);

Parameters

| eProductModules | The product modules that should be started. |
| --- | --- |
| pHandle | Pointer to a memory location the will receive a HANDLE that is signaled when the product is stopped. |

Remarks

This function starts the full product or a specified product option. Possible values for eProductModules are:

eProductModulesAll – The full product is started.

eProductModulesRoutingRuntime – The ‘routing module’ is started. Parts of an eventually installed ISDN Access Server that are not necessary for LAN targets or internal administrated users may not be started. Any product modules not needed at runtime (e.g. configuration and monitoring modules) may not be started.

eProductModulesRuntime – All ‘runtime modules’ are started to provide full product features. Any product modules not needed at runtime (e.g. configuration an monitoring modules) may not be started.

eProductModulesAdmin – Starts the configuration and monitoring modules.

eProductModulesRoutingRuntimeAndAdmin – Starts the ‘routing module’ together with configuration and monitoring modules.

When the function returns, it is not guaranteed that the product modules are fully initialized and will fully initialize without failure.

No process should have the configuration open when this function is called.

NetWAYS/ISDN specific:

The program NETWAYS.EXE will be executed. A loaded NetWAYS/ISDN driver without NETWAYS.EXE running is a not running product.

The NetWAYS/ISDN service available in Windows 95 and Windows NT is not affected by this function. All values of eProductModule are internally mapped to eProductModulesAll.

It makes no sense to use this function when the NetWAYS/ISDN service is used, since the service will start NETWAYS.EXE automatically.

ISDN Multiprotocol Router and ISDN Access Server specific:

All values defined above for eProductModules are supported. The following table illustrates the current use of the flags:

| eProductModules | Service name |  |  |  |
| --- | --- | --- | --- | --- |
|  | NTMPRI | AVMWEBS | NTREUD | NTREACT |
| eProductModulesAll | Started | Started | Started | Started |
| eProductModulesRoutingRuntime | Started | not started | not started | not started |
| eProductModulesRuntime | Started | not started | Started | Started |
| eProductModulesAdmin | not started | Started | not started | not started |
| eProductModulesRoutingRuntimeAndAdmin | Started | Started | not started | not started |

Return Value

|  | AVMNW_OK | All specified product modules have been started or are already running. |  |
| --- | --- | --- | --- |
|  | AVMNW_ERROR_FAILED | At least one of the specified product modules is not running and could not be started. |  |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR |  | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported, see above

NetWAYS for Windows NT – supported, see above

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

AVMNW_Module_Stop

AVMNW_Module_GetStatus

AVMNW_RESULT PASCAL AVMNW_Module_Stop(
   enum AVMNWProductModules eProductModules);

Parameters

| eProductModules | The product modules that should be stopped. |
| --- | --- |

Remarks

This function stops all or part of the product modules.

When the function returns it is not guaranteed that the product modules are fully stopped and will fully stop without failure.

If the product modules are running and a object handles have been acquired by AVMNW_Module_Start(), the object will be signaled when the product module actually terminates.

NetWAYS/ISDN specific:

All values of eProductModule are internally mapped to eProductModulesAll. The NetWAYS/ISDN service available in Windows 95 and Windows NT is not affected by this function.

ISDN Multiprotocol Router and ISDN Access Server specific:

Only the specified modules are terminated.

Return Value

|  | AVMNW_OK | All requested product modules are stopping or are not running. |  |
| --- | --- | --- | --- |
|  | AVMNW_ERROR_FAILED | At least one running product module couldn’t be stopped. |  |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR |  | General error (e.g. not registered) |

Product Specifics

NetWAYS for Windows 95 – supported, see above

NetWAYS for Windows NT – supported, see above

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

AVMNW_Module_Start

AVMNW_RESULT PASCAL AVMNW_Module_GetStatus(
   enum AVMNWProductModules eProductModules,
   /*OUT*/char *szVersion, /*INOUT*/unsigned int *pnVersionSize,
   /*OUT*/char *szRunInfo, /*INOUT*/unsigned int *pnRunInfoSize);

Parameters

| eProductModules | The product module the state is requested for. |
| --- | --- |

Remarks

This function delivers the running status of the requested product option. The values for eProductModules are the same as in AVMNW_Module_Start().

The product modules are defined to be running, when all product modules are running that would be started by AVM_Module_Start() when called with the same value for eProductModules.

If AVMNW_OK is returned, a version information will be returned in szVersion and a runtime info (to ‘be sure’ that the modules are really running) is returned in szRunInfo. The sizes pnVersionSize and pnRunInfoSize are handled like in AVMNW_Config_Attribute_GetText(). Both information are human readable strings for display purposes.

NetWAYS/ISDN specific:

All values for eProductModules are internally mapped to eProdcutModulesAll.

ISDN Multiprotocol Router and ISDN Access Server specific:

All values defined in AVM_Start() for eProductModules are supported.

Return Value

|  | AVMNW_OK | The requested product modules are running. |  |
| --- | --- | --- | --- |
|  | AVMNW_ERROR_PARTLY | Not all of the specified product modules are running. At least one of the specified product modules is running. |  |
|  | AVMNW_ERROR_NONE | None of the specified product modules is running. |  |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR |  | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported, see above

NetWAYS for Windows NT – supported, see above

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

AVMNW_Module_Start

AVMNW_Config_Attribute_GetText

AVMNW_RESULT PASCAL AVMNW_PasswordProvider_Register(
   BOOL bRegister, 
   DWORD dwFlags,
   /*OUT*/HANDLE *phSema,
   HWND hWnd, UINT msg);

Parameters

| bRegister | Tell if you want to register or unregister as a password provider. |
| --- | --- |
| dwFlags | Reserved for future use. Set to 0 always. |
| phSema | Pointer to a memory location that will receive a semaphore handle to be signaled when passwords are required. May be NULL if no semaphore required. |
| hWnd | Window handle to be posted a message when passwords are required. May be NULL. |
| msg | Message to be posted to hWnd. |

Remarks

Call this function to register as a password provider. A password provider is a software module the provides usernames and password for the AVM product. The username/password pairs will no longer be taken from the static database.

The password provider calls AVMNW_PasswordProvider_Register with bRegister = TRUE first. The function returns immediately.

If phSema or hWnd are provided in that call, the password provider will be notified every time the AVM product requires passwords. If phSema is not NULL a semaphore is signaled. If phSema is NULL and hWnd is not NULL, the message msg is posted to hWnd.

Whenever the password provider is notified, it should call AVMNW_PasswordProvider_Callback to tell the API that is ready to provide the password now. Before this function returns, the API calls a callback function in the password provider which actually handles password delivery.

There is maximum time that may expire from notification to password delivery. The time is about 20 seconds long.

A registered password provider have to unregister using bRegister = FALSE before it terminates.

It is allowed to register as a password provider before the product is running.

The product guarantees that passwords delivered by the password provider are not stored in the file system. The password provider is called every time access to a password is required. Stored copies in memory are physically cleared as soon as possible.

Return Value

|  | AVMNW_OK | The function succeeded |
| --- | --- | --- |
|  | AVMNW_ERROR_BUSY | There is already another password provider registered. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

See Also

AVMNW_PasswordProvider_Callback

AVMNW_PASSWORDCALLBACK

AVMNW_RESULT PASCAL AVMNW_PasswordProvider_Callback(
   void *pPrivate,
   AVMNW_PASSWORDCALLBACK pCallbackFunction);

Parameters

| pPrivate | Parameter that is transparently transferred to the callback function. |
| --- | --- |
| pCallbackFunction | The callback function to be called by the API to handle password delivery. |

Remarks

This function tells the API to do the callback for password delivery. The registered password provider should call this function every time it is notified from the AVM product that passwords are required.

Within this function the API will call the callback specified by the pCallbackFunction parameter.

Since password providing is time critical, this function should be called as soon as possible after notification. Any user interaction should be avoided.

Return Value

|  | AVMNW_OK | The function succeeded. |  |
| --- | --- | --- | --- |
|  | AVMNW_ERROR_PARAM |  | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR |  | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported

ISDN Access Server – supported

See Also

AVMNW_PasswordProvider_Register

AVMNW_PASSWORDCALLBACK

typedef void (PASCAL * AVMNW_PASSWORDCALLBACK)(
   void *pPrivate,
   struct AVMNWPasswordProviderParameters *pParameters );

Parameters

| pPrivate | Parameter that is transparently transferred from AVMNW_PasswordProviderCallback. |
| --- | --- |
| pParameters | Pointer to the exchanged usernames/passwords. |

Remarks

This function is implemented by the registered password provider and called by the API from within AVMNW_PasswordProviderCallback. The parameter pParameters points to the following structure:

struct AVMNWUsernameParameters {
/*INOUT*/	char szUsernameInbound[AVMNW_MAX_USERNAME+1];
/*INOUT*/	char szUsernameOutbound[AVMNW_MAX_USERNAME+1];
};

struct AVMNWPasswordParameters {
/*IN*/		BOOL bInboundAuthentication; // FALSE: it is outbound authentication
/*INOUT*/	char szPassword[AVMNW_MAX_PASSWORD+1];
};

struct AVMNWPasswordProviderParameters {
/*IN*/		unsigned int nSize;	// sizeof(AVMNWPasswordProviderParameters)
/*IN*/		unsigned long InternalHandle; // API Internal Info – don’t modifiy
/*IN*/		char szTargetName[AVMNW_MAX_TARGETNAME+1];
/*IN*/		unsigned int nController;

/*IN*/		BOOL bUsernameQuery;
			union {
				struct AVMNWUsernameParameters u; // if bUsernameQuery
				struct AVMNWPasswordParameters p; // if !bUsernameQuery
			};
};
#pragma pack()

The element bUsernameQuery indicates the mode of the callback: Either username (if TRUE) or password providing (if FALSE).

The element szTargetName is the name of the target a password or usernames are required for.

The element nController specifies the CAPI 2.0 controller number on which the connection is established.

In case of password providing the substructure p is valid:

bInboundAuthentication tells if it is an inbound or outbound authentication.

Before the callback returns it can modify the element szPassword. The product will clear the password in memory immediately after use and will call the password provider every time a password is needed.

In case of username providing the substructure u is valid:

Before the callback returns it can modify the element szUsernameInbound (used for inbound authentication) and/or szUsernameOutbound (used for outbound authentication). As long as a physical connection exists the product may keep the usernames and doesn’t have to call the password provider more than once.

Since password providing is time critical, the callback function should return as soon as possible. It should avoid to do any user interaction.

NetWAYS/ISDN specific:

From the notification to password delivery, the NetWAYS/ISDN user interface will not process any user input or windows messages.

NT/MPRI and ISDN Access Server specific:

For incoming calls any modifications of szUsernameInbound for inbound authentication may be ignored. (The calling target is identified by the inbound username configured in the database, before the password provider is queried.)

Return Value

There is no return value.

Product Specifics

NetWAYS for Windows 95 – supported, see above

NetWAYS for Windows NT – supported, see above

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

AVMNW_RESULT PASCAL AVMNW_Connection_Authenticate(
   char *szTargetName);

Parameters

| szTargetName | Name of the target for which the connection will be authenticated. |
| --- | --- |

Remarks

This function authenticates the remote peer of all existing physical connections (established LCP links) via CHAP.

If no (physical) connection exists to the remote peer specified by szTargetName, nothing is done.

It is required that the inbound authentication for the target is set to CHAP.

No indication about successful or failed authentication will be given by this function. If the authentication fails, the physical connections will be disconnected.

If there is a registered password provider, it will be called to provide the password for the inbound authentication.

NetWAYS/ISDN specific:

Only PPP targets are supported.

NT/MPRI and ISDN Access Server specific:

The szTargetName has to be a LAN target or internal administrated user.

Return Value

|  | AVMNW_OK | CHAP inbound authentication will be done on all physical connections (if any). |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The given target was not found or has not set CHAP for inbound authentication. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

AVMNW_RESULT PASCAL AVMNW_Connection_InvalidateSessionKey(
   char *szTargetName);

Parameters

| szTargetName | Name of the target for which the current session key for ECP will be invalidated. If this parameter is NULL or empty, all keys for all connections are invalidated. |
| --- | --- |

Remarks

This function invalidates the current ECP (encryption control protocol) session key and establishes a new one. If no target is given, the keys for all encrypted connections are  invalidated. The session key used for outgoing data is changed. To change the session keys for both directions, both sides must call this function.

If a connection is suspended at the time the function is called,  the session key will be changed when the connection is reactivated. The function call itself will not reactivate a suspended connection. For a logically disconnected connection nothing is done, since a new session key is always used when a new logical connection is established.

No indication about success or failure will be given by this function. If the change of the session key fails, the connection will be disconnected.

A registered crypt provider is used to secure the transport of the new session key.

NetWAYS/ISDN specific:

Only PPP targets are supported.

NT/MPRI and ISDN Access Server specific:

The szTargetName has to be a LAN target or internal administrated user (if not empty).

Return Value

|  | AVMNW_OK | The session key will be changed. |
| --- | --- | --- |
|  | AVMNW_ERROR_NOTFOUND | The given target was not found or has no data encryption configured. |
|  | AVMNW_ERROR_PARAM | At least one of the input parameters is illegal. |
|  | AVMNW_ERROR | General error (e.g. not registered). |

Product Specifics

NetWAYS for Windows 95 – supported

NetWAYS for Windows NT – supported

NT/MPRI – supported, see above

ISDN Access Server – supported, see above

See Also

## Object Schema

## Object Types and Attributes

The following object types are defined:

| Name | #define | Description |
| --- | --- | --- |
| Product | AVMNW_ObjectType_Product | Single instance object that hold the global product configuration |
| Controller | AVMNW_ObjectType_Controller | Object representing an installed ISDN controller. |
| Target | AVMNW_ObjectType_Target | Object holding target information (e.g. number to call) |
| IPWANRoute | AVMNW_ObjectType_IPWANRoute | Static IP route associated to a target. |
| IPXWANRoute | AVMNW_ObjectType_IPXWANRoute | Static IPX WAN route associated to a target. Note that these object can’t be controlled by the current web pages. What is called ‘IPX WAN route’ on the web pages is actually a SAPService object. |
| SAPService | AVMNW_ObjectType_SAPService | Static SAPService (combined with a static IPX RIP route) associated to a target. |
| IPXSAPFilterSet | AVMNW_ObjectType_IPXSAPFilterSet | A set of IPX SAP filter rules. |
| IPXSAPFilterRule | AVMNW_ObjectType_IPXSAPFilterRule | A SAP filter rule |

Each object of a specific type has a defined set of attributes. Each attribute has a data type.

## Attribute types

Helper definitions

| BOOL | as defined in windows.h |
| --- | --- |
| DWORD | as defined in windows.h |
| BYTE | as defined in windows.h |
| AVMNW_IP_ADDRESS | DWORD (e.g. 0x7f000001 for 127.0.0.1) |
| AVMNW_IP_MASK | BYTE (0..32) |

Attribute types

| Text | char[], ‘\0’ terminated, single byte ASCII character set |
| --- | --- |
| Number | unsigned int (32 bit) |
| Bool | BOOL |
| IPRoute | #pragma pack(4) struct AVMNW_IPRoute { unsigned int nSize; // sizeof(IPRoute) BOOL bActive; // activated? AVMNW_IP_ADDRESS TargetIPAddress; AVMNW_IP_MASK TargetIPMask; DWORD Metric; }; #pragma pack() |

## Object types

Each attribute have a definition in avmnwapi.h with AVMNW_Attrib_ as a prefix for the attribute name (e.g. AVMNW_Attrib_szMSN for the szMSN attribute).

Product

| Attribute | Type | Valid Values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| bCLICheck | Bool |  | Toggles CLI security |
| szFirstCLI | Text | digits or * or ?, 0..23 chars | First configured CLI (for CLI check and/or CLI-assignment to a target). Set it to an empty text to delete the CLI. |
| nSAPInputFilterSetID | Number |  | ID of the active inbound IPX SAP Filter Set Object. 0: no active |
| nSAPOutputFilterSetID | Number |  | ID of the active outbound IPX SAP Filter Set Object. 0: no active |
| nMaxMonthlyCharge | Number |  | Max. allowed monthly charge in 1/1000 of configured currency. 0 means disabled maximum. |
| nMaxWeeklyCharge | Number |  | Max. allowed weekly charge in 1/1000 of configured currency. 0 means disabled maximum. |
| nMaxDailyCharge | Number |  | Max. allowed daily charge in 1/1000 of configured currency. 0 means disabled maximum. |
| nMaxMonthlyPhysicalUpTime | Number |  | Max. allowed monthly physical up time in seconds. 0 means disabled maximum. |
| nMaxWeeklyPhysicalUpTime | Number |  | Max. allowed weekly physical up time in seconds. 0 means disabled maximum. |
| nMaxDailyPhysicalUpTime | Number |  | Max. allowed daily physical up time in seconds. 0 means disabled maximum. |
| nMaxMonthlyOutgoingCalls | Number |  | Max. allowed monthly number of outgoing calls. 0 means disabled maximum. |
| nMaxWeeklyOutgoingCalls | Number |  | Max. allowed weekly number of outgoing calls. 0 means disabled maximum. |
| nMaxDailyOutgoingCalls | Number |  | Max. allowed daily number of outgoing calls. 0 means disabled maximum. |
| bNoWanWanRouting | Bool |  | TRUE: WAN to WAN (ISDN to ISDN) routing is disabled. FALSE: WAN to WAN (ISDN to ISDN) routing is enabled. |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 1 (deletion/creation not supported) |
| ISDN Access Server | 1 (deletion/creation not supported) |

##### Controller

| Attribute | Type | Valid Values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| nCAPI20Controller Objects can be ‘used’ by this attribute. | Number | 1..4 |  |
| szMSN | Text | digits, 0..23 chars | For 1TR6 controllers this should contain the single digit EAZ. |
| bPBX | Bool |  | Connected via a PBX? |
| szOutsideLineAccess | Text | digits, 0..4 chars | Dial prefix to reach sites outside the PBX (Only used if connected via a PBX.). |
| nLengthOfOutsideNumbers | Number |  | Minimal length of numbers that will be automatically preceeded with the szOutsideLineAccess for dial out. (Only used if connected via a PBX.) |
| bIgnoreIncomingCalls | Bool |  | If TRUE, all incoming calls on this controller are ingored. Other applications can accept data connections even if they operate with the same MSN. |
| szNumber | Text | digits, 0..24 chars | Own ISDN number. Used for AVM proprietary protocol and for multilink PPP (for building the Endpoint Descriminator). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| SzPBXExtension | Text | digits, 0..7 chars | The own PBX extension (if connected to a PBX). Used for AVM proprietary protocol and for multilink PPP (for building the Endpoint Descriminator). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| szAreaCode | Text | digits, 0..7 chars | The own area code. Used for AVM proprietary protocol and for multilink PPP (for building the Endpoint Descriminator). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| szCountryCode | Text | digits, 0..3 chars | The own country code. Used for AVM proprietary protocol and for multilink PPP (for building the Endpoint Descriminator). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| szIntDialPrefix | Text | digits, 0..4 chars | The own international dialing prefix. Used for AVM proprietary protocol and for multilink PPP (for building the Endpoint Descriminator). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| nRetryNumber | Number |  | Number of redial attempts. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| bRetryUnlimited | Bool |  | Specifies unlimited redial attempts. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| nRetryPause | Number |  | Pause in seconds between redial attempts. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| bRetryCountNotAll | Bool |  | Specifies if redial attempts that caused no charges are counted for comparision with nRetryNumber to terminate redail. FALSE: All redial attempts are counted. TRUE: Only redial attempts that caused charges (that means established B-Channel) are counted as redial attempts. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| bRetryOnConnectionerror | Bool |  | Specifies if NetWAYS keeps the standby state on authentication errors. FALSE: NetWAYS will not keep the standby state on authentication errors. Manual user interaction is required to connect. TRUE: NetWAYS keeps the standby state on authentication errors and enters a pause state before further attempts are made to get a connection. See nRetryOnConnectionerrorPause for the length of the pause. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| nRetryOnConnectionerrorPause | Number | Should be a multiply of 60 | Specifies the length in seconds of the pause state after authentication errors (if bRetryOnConnectionerror is TRUE). NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| nMaxMonthlyCharge | Number |  | Max. allowed monthly charge in 1/1000 of configured currency. 0 means disabled maximum. |
| nMaxMonthlyPhysicalUpTime | Number |  | Max. allowed monthly physical up time in hours. 0 means disabled maximum. |
| nMaxMonthlyOutgoingCalls | Number |  | Max. allowed monthly number of outgoing calls. 0 means disabled maximum. |

Number of Objects:

| NetWAYS/ISDN | 1 (deletion/creation not supported) |
| --- | --- |
| NT/MPRI | 0...4 |
| ISDN Access Server | 0...4 |

Target

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. NetWAYS/ISDN specific: Attribute not supported |
| szName Objects can be ‘used’ by this attribute. | Text |  | The name of the target. Can be set in precreation state only. |
| szISDNNumber | Text | 23 | ISDN dial string to reach the remote site. NetWAYS/ISDN specific: See also: szCountryCode, szAreaCode, szProviderCode |
| szOutboundUsername | Text | 47 | The szOutboundUsername (alias local system id) is used for authentication at the remote site (outbound authentication). |
| szOutboundPassword | Text | 47 | The szOutboundPassword is used in conjunction with the szOutboundUsername for authentication at the remote site (outbound authentication). |
| nInboundAuthentication | Number |  | AVMNW_Auth_Disabled The remote site will not be authenticated. AVMNW_Auth_PAP The remote site will be authenticated using PAP (clear text passwords on the line). AVMNW_Auth_CHAP The remote site will be authenticated using CHAP (crypted passwords on the line). |
| szInboundUsername | Text | 47 | The szInboundUsername (remote system id) is used in conjunction with the szInboundPassword for authentication of the remote site (inbound authentication). It may also be used for identification of incoming connections. |
| szInboundPassword | Text | 47 | The szInboundPassword is used in conjunction with the szInboundUsername (remote systemid) for authentication of the remote site (inbound authentication). |
| BActive | Bool |  | This attribute actives/deactivates the a target. NetWAYS/ISDN specific: If TRUE, the NetWAYS/ISDN autoconnect/standby mode will be set for this target. If FALSE, the autoconnect mode will disabled. |
| nActiveMode | Number |  | NetWAYS/ISDN specific: The mode of the autoactivation if bActive is TRUE: ### AVMNW_ActiveMode_ConnectAtServiceStart A connection is established when the service is started. ### AVMNW_ActiveMode_StandbyAtServiceStart The standby mode is entered when the service is started. ### AVMNW_ActiveMode_ConnectAtGUIStart A connection is established when the GUI is started. ### AVMNW_ActiveMode_StandbyAtGUIStart The standby mode is entered when the GUI is started. |
| szDNSExclusivePattern | Text | 63 | Only DNS request with names that match this pattern are send via this target. An empty text means that DNS filtering disabled. |
| FirstStaticIPRoute | IPRoute |  | The first static IPRoute defined for the target. NetWAYS/ISDN specific: Attribute not supported |
| nShortHoldMode | Number |  | AVMNW_ShortHold_Disabled short hold deactivated AVMNW_ShortHold_Profile short hold via charge profile AVMNW_ShortHold_Inactivity short hold via fixed inactivity timeout AVMNW_ShortHold_Selflearning short hold based on ISDN charge infos |
| nInactivityTimeout | Number |  | Number of Seconds for InactivityTimeout (if nShortHoldMode is set to AVMNW_ShortHold_Inactivity) |
| nDisconnectMode | Number |  | AVMNW_Disconnect_Disabled log. disconnect deactivated (‘disconnect never’) AVMNW_Disconnect_Immediately log. disconnect with phys. disconnect AVMNW_Disconnect_Timeout log disconnect later than phys. disconnect |
| nDisconnectTimeout | Number |  | Number of Minutes for disconnect timeout (if nDisconnectMode is set to AVMNW_Disconnect_Timeout) |
| bIPMasquerade | Bool |  | Toggles activation of IP-Masquerading |
| IPMasqueradeAddress | Number |  | IP-Address to be used for IP-Masquerading (use 0.0.0.0 for dynamical assignment) The coding is like in AVMNW_IP_ADDRESS NetWAYS/ISDN specific: The local IP-Address even if masquerading is disabled. |
| bIPMasqueradeIncoming | Bool |  | Specifies if masquerading supports incoming connections in NetWAYS. bIPMasquerade must be set to TRUE. NT/MPRI and ISDN Access Server Specific: Attribute not supported. |
| bIPActivated | Bool |  | Toggles activation of IP. Note that all static IP routes of this target will be deleted if IP is deactivated. |
| bIPXActivated | Bool |  | Toggles activation of IPX |
| bCLICheck | Bool |  | Toggles CLI check. NT/MPRI and ISDN Access Server Specific: Attribute not supported (use attributes of the Product Object) |
| szCLI | Text | 23 digits | CLI for CLI check (if bCLICheck is TRUE) NT/MPRI and ISDN Access Server Specific: Attribute not supported (use attributes of the Product Object) |
| nUseIPGateway | Number |  | This attribute defines gateway usage when using dynamic IP address assignment. If a gateway is learned by the system a ‘default route’ is added for NetWAYS/ISDN. AVMNW_UseIPGateway_WhenConnected The gateway is learned only when connected. AVMNW_UseIPGateway_Always The gateway is learned even in standby mode. ### AVMNW_UseIPGateway_Never The gateway is learned never. NT/MPRI and ISDN Access Server specific: Attribute not supported |
| bSNMPOverIPFilter | Bool |  | Toggles the SNMP over IP filter. |
| bSNMPOverIPXFilter | Bool |  | Toggles the SNMP over IPX filter. |
| NDataEncryption | Number |  | #### AVMNW_Encryption_Disabled #### Data encryption will be disabled. #### AVMNW_Encryption_Twofish The data will be encrypted using the Twofish algorithm using key size 128 bit in ECP. #### AVMNW_Encryption_Twofish_192 The data will be encrypted using the Twofish algorithm using key size 192 in ECP. #### AVMNW_Encryption_Twofish_256 The data will be encrypted using the Twofish algorithm using key size 256 in ECP. |
| bRemoteNode | Bool |  | #### If set to TRUE this target represents an internally administered user in the ISDN Access Server product. If set to FALSE this target represents a LAN-LAN target of the NT/MPRI product. This attribute is currently read only. NetWAYS/ISDN specific: Attribute not supported |
| szCountryCode | Text | 3 digits | The country code of the target. The ISDN number to reach the target can be optionally devided into its components szCountryCode, szAreaCode, szProviderCode and szISDNNumber to make use of the location features of NetWAYS/ISDN. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| szAreaCode | Text | 7 digits | The area code of the target. The ISDN number to reach the target can be optionally devided into its components szCountryCode, szAreaCode, szProviderCode and szISDNNumber to make use of the location features of NetWAYS/ISDN. NT/MPRI and ISDN Access Server Specific: Attribute not supported |
| szProviderCode | Text | 7 digits | The number of the ISDN provider to be used (e.g. 01033 for Deutsche Telekom when calling from Germany). The ISDN number to reach the target can be optionally devided into its components szCountryCode, szAreaCode, szProviderCode and szISDNNumber to make use of the location features of NetWAYS/ISDN. NT/MPRI and ISDN Access Server Specific: Attribute not supported |

Number Of Objects:

| NetWAYS/ISDN | 0...virtual unlimited |
| --- | --- |
| NT/MPRI | 0..>30 |
| ISDN Access Server | 0 |

IP WAN Route

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| nTargetID | Number |  | The Target the IP WAN Route belongs to. Can be set in precreation state only. |
| IPRoute DWORD Metric; | IPRoute |  | The static route. |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 0..virtually unlimited |
| ISDN Access Server | 0 (deletion/creation not supported) |

IPX WAN Route

Generates a static IPX route entry at runtime.

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| nTargetID | Number |  | The Target the IP WAN Route belongs to. Can be set in precreation state only. |
| IPXNetAddress | Number |  | The IPX net address routed for the target. (e.g. the decimal value 255 for the IPX net address ‘000000FF’) |
| nHopCount | Number |  | The hop count. |
| nTicks | Number |  | The ticks value. |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 0..virtually unlimited |
| ISDN Access Server | 0 (deletion/creation not supported) |

SAP Service

Generates a static SAP and IPX route entry at runtime. The node address for the SAP entry is always 00000000001.

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| nTargetID | Number |  | The Target the IP WAN Route belongs to. Can be set in precreation state only. |
| IPXNetAddress | Number |  | The IPX net address of the SAP service. (e.g. the decimal value 255 for the IPX net address ‘000000FF’) |
| nServiceType | Number |  | The service type (e.g. 4 for ‘File Server’) |
| szServiceName | Text | no zero length string allowed | The name of the service. |
| nSocket | Number |  | The IPX socket of the service. |
| nHopCount | Number |  | The hop count. |
| nTicks | Number |  | The ticks value. |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 0..virtually unlimited |
| ISDN Access Server | 0 (deletion/creation not supported) |

IPX SAP Filter Set

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| bIsInputFilter | Bool |  | Specifies the direction of the filter. FALSE: It is an output filter. TRUE: It is an input filter. |
| bDefaultPolicyAccept | Bool |  | Specifies the default action if no filter rule matches a service: FALSE: The service is filtered. TRUE: The service passes the filter. |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 0..virtually unlimited |
| ISDN Access Server | 0..virtually unlimited |

IPX SAP Filter Rule

| Attribute | Type | Valid values | Note |
| --- | --- | --- | --- |
| nID Objects can be ‘used’ by this attribute. | Number |  | The object ID. Get only. |
| nIPXSAPFilterSetID | Number |  | The IPX SAP Filter Set this rule belongs to. Can be set in precreation state only. |
| bPolicyAccept | Bool |  | Specifies the action if the filter rule matches a SAP service: FALSE: The service is filtered. TRUE: The service passes the filter. |
| szServiceName | Text | no zero length string allowed | The name of the service to be filtered. |
| nServiceType | Number |  | The type of the service to be filtered. |
| nTargetID | Number |  | 0: All Targets |

Number of Objects:

| NetWAYS/ISDN | 0 (Object not supported) |
| --- | --- |
| NT/MPRI | 0..virtually unlimited |
| ISDN Access Server | 0..virtually unlimited |