PCjr ROM-CLOCK installation instructions.

TOOLS REQUIRED:

- 28 pin IC extraction tool

(or small flat-blade screwdriver, or pocket knife)

- Medium flat-blade screwdriver

HARDWARE INSTALLATION  (For excellent diagrams refer to the 'Diskette Drive

Installation Instructions' in your 'Guide to Operations' manual):

1.  Turn off power to PCjr and all peripherals (printer, display, etc.)

2.  Unplug the PCjr's and peripherals' power cords from the wall outlets.

3.  Turn the PCjr around so the rear is facing you.

4.  Wait five minutes to allow the PCjr to cool down.

WARNING: Serious shock hazards exist inside the PCjr.  Make sure the

power cord is unplugged before you remove the cover.

5.  Using the medium screwdriver, remove PCjr top cover.  (See diagrams in

the 'Guide to Operations' manual)

6.  Turn the PCjr around so the front is facing you.

7.  Remove power and signal cables from the back of the diskette drive.  (See

diagrams in the 'Guide to Operations' manual)

8.  Remove diskette drive from PCjr by pulling the rear of the diskette drive

out of the mounting holes and sliding the drive out of the front panel

slot.  This may take a fair amount of effort, and the drive has some sharp

parts which can induce a lot of pain if care is not taken.	(See diagrams

in the 'Guide to Operations' manual)

9.  Locate the ROMs located just behind the left cartridge slot.  The ROMs

are the 28 pin ICs that are in sockets.  The second ROM (one closest to

the rear) is the ROM that the ROM-CLOCK will sit under.

back

__________________________________________________

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|						       |

|			       ---------	       |

|	This is the ROM  ==>  | 	|	       |

|	you need to find      |o	|	       |

|			       ---------	       |

|						       |

|			       ---------	       |

|			      | 	|	       |

|			      |o	|	       |

|			       ---------	       |

|						       |

|			      =========== ============ |

|			       left slot   right slot  |

|			     __________________________|

|_______________________|

front

WARNING: All parts are sensitive to static discharge.  Follow the static

discharge handling rules.  (Don't perform work in carpeted areas, don't

excessivly handle device leads, etc.)

10. Remove the ROM identified above using the 28 pin IC extraction tool

(the orientation of this ROM is very important, so you may place a small

mark on the ROM, so you can put it back in the same direction.)  Place

the ROM in the back side of the ROM-CLOCK's conductive foam.  (If you

are using the screwdriver or pocket knife, pry the chip up very gently,

and evenly so as to NOT bend the ROM's pins)

11. Orient the ROM-CLOCK in the proper direction ('E' printed in the socket

circuit board, located on the right hand side), and press firmly into

the ROM's socket.

ROM's original orientation:

back of jr

|						       |

|			       ---------	       |

|			      | 	|	       |

|			      |o	|	       |

|			       ---------	       |

|						       |

front of jr

ROM-CLOCK orientation:

back of jr

|						       |

|			       ---------	       |

|			      |  ____	|	       |

|			      | |o___| E|	       |

|			       ---------	       |

|						       |

front of jr

12. Orient the ROM in the proper direction (the same way as when you removed

it, and the same direction as the ROM just below it-- the one next to the

left cartridge slot), and press it firmly into the ROM-CLOCK socket.

13. Install the diskette drive.  (See the 'Guide to Operations' manual)

14. Reconnect the power and signal cables to the diskette drive.  (See the

'Guide to Operations' manual)

15. Put the top cover back onto the PCjr.  (See the 'Guide to Operations'

manual)

16. Plug the power cords back into the wall sockets.