There is an error, (calling programming errors "bugs" tends to suggest

they are cute little annoyances instead of programming blunders)

a blunder in 3.x which causes floppy drives to have a "grinding" sound

each time the drive is accessed.  This problem does not exist in 2.x.

The grinding sound is caused by an incorrect step rate.  The speed at

which the disk head is moved is incorrectly set on booting.  The

solution is to reset the step rate for floppy drives which can be done

easily if you happen to know what to do.  The correct step rate is

already part of DOS 3.x and through the use of the AX register and

Interrupt 13 the correction can be made.  All that is needed is to set

the AX register to zero with the instruction "xor ax,ax" and then call

for "int 13."  This tiny program needs to end with "int 20" to return

your system to DOS.

xxxx:0100    xor  ax,ax

xxxx:0102    int  13

xxxx:0104    int  20

You can create this program with DEBUG or simply use the one that

accompanies this explanation.  Insert this tiny COM file in your

AUTOEXEC.BAT file and your floppies, when using 3.x, will purr

like a kitten.

C. J. Handlogen

9235 Whitehurst

Dallas, TX 75243

P. S.  If you find any errors in this information, please refer to them

as "bugs."  All other errors are "blunders."  Thanks.