INT10FIX DOCUMENTATION

Published by:

PC Enterprises

2400 Belmar Blvd,  Wall NJ  07719

Authors: P. Joseph Calabria and Kurt J. Stampone

A small percentage of jrExcellerator customers have reported

that their PCjr occasionally beeps and ignores key-presses

when the jrExcellerator is used in "fast" mode.  Although this

problem is encountered by very few PCjr users, it is a

definite problem that has a solution.

The problem is caused by improperly written code in the

PCjr's ROM BIOS.  That's why replacing the jrExcellerator can

not correct this problem.  Instead, the solution is to use the

FIXBEEP program to correct the PCjr's ROM BIOS.  If you are

not experiencing a keyboard beeping problem then there is no

benefit in running the FIXBEEP program.

The keyboard beeping problem is caused by bugs in the PCjr's

ROM BIOS routine used to handle keyboard input.  The BIOS

routine incorrectly uses the Intel 8253 timer chip which is

decodes keyboard transmissions.  Intel, the manufacturer of

the timer chip, specifies that the chip must be allowed at

least one microsecond to settle after each read

or write operation.  The PCjr's ROM BIOS clearly violates this

specification.

In most cases, Intel's timer chips are actually much better

than their specifications indicate.  That's why the violation

rarely causes a problem when the PCjr is operated at 4.77MHz.

However, when the PCjr is operated at 9.54MHz, these bugs

prevent some of Intel's timer chips from working properly.

The FIXBEEP program provides the solution.  It allows

the PCjr to comply with the timer chip's specifications by

allowing a microsecond wait after reading or writing to the

chip.  The FIXBEEP program has been placed in the Public

Domain and we encourage you to share it with other PCjr users

who encounter a keyboard beeping problem.

If you are using the PC Enterprises Keyboard Buffer Cartridge

be sure to run the JRSYNC or JRSYNCNP program before you run

FIXBEEP.

To use the FIXBEEP program we recommend you copy the program

to your boot disk and include the FIXBEEP /I command in your

AUTOEXEC.BAT file.  By doing this the keyboard beeping

corrections will be made automatically each time you boot

your computer.

FIXBEEP must be run from high memory (memory above 128KB)

because it is timing dependent.  If FIXBEEP is not run from

high memory, the wait states added by the PCjr's video

circuitry would prevent your keyboard from working.  To

avoid this problem, if you attempt to install FIXBEEP in low

memory, the program will refuse to load, and a message will

tell you why.

If you receive the  "Program is not being loaded into

fast memory" message, it means you tried to load FIXBEEP in

slow (internal) memory.  There are at least two ways you can

correct this problem.

One method that will force FIXBEEP, and all your software

to load in the PCjr's "fast" memory is to modify the command

you are using to install your memory management software.  If

you are use jrConfig this means do not use the -S option at

all, or use the -S option with a value of 91 or larger.

For a quick and easy solution, simply use the jrExcellerator's

FASTMEM command before you run FIXBEEP.  FASTMEM will not do

anything at all if all of your PCjr's slow memory has already

been set aside.  However, if slow memory is still available,

FASTMEM will set it aside for you.  You may wish to include

the FASTMEM command in your AUTOEXEC.BAT file, as long as it

appears before FIXBEEP.

If your jrExcellerator ROM BIOS is revision 3.0 or later the

FIXBEEP program is built into the jrExcellerator.  This offers

several advantages.  First, the FIXBEEP command becomes an

internal DOS command, which means it never needs to be on your

diskette drive.  Second, the version of FIXBEEP built into the

jrExcellerator BIOS, always runs from ROM instead of RAM.  In

other words, you'll never need to worry about slow memory

because programs built into ROM run faster than programs which

execute from RAM.

Unfortunately, jrExcellerators that use an earlier ROM BIOS

can not be upgraded without returning the jrExcellerator to

PCE.  The reason is that the FIXBEEP program requires a larger

EPROM and modifications must be made to the jrExcellerator

to use the new EPROM.

The diskette version of FIXBEEP requires the /I option before

it will install.  To install FIXBEEP type: FIXBEEP /I

To see the help screen type: FIXBEEP /?

In addition to these options FIXBEEP also provides parameters

for diagnostic test purposes.  In the unlikely event that

FIXBEEP does not correct a beeping problem that you

experience, install FIXBEEP using the /D or /S parameters.  In

this mode, any time a beep is produced, FIXBEEP will display

information PCE can utilize to determine what caused the beep.

If this happens, write down any codes produced in diagnostic

mode, then contact PCE.  (Remember: It is normal for the PCjr

to beep while the diskette drive light is on unless you have

a Keyboard Buffer Cartridge installed).

For more information contact:

PC Enterprises

"The jrProducts Group"

Customer Service: 1-908-280-0025

Order and Catalog Hotline: 1-800-922-PCjr

The PCjr Lives

There is a Reason!