INT10FIX DOCUMENTATION

Published by:

PC Enterprises

2400 Belmar Blvd,  Wall NJ  07719

Author: P. Joseph Calabria

A known incompatibility exists with the PCjr's INT 10 routine. The

problem concerns several modes, however, problems with the 620x200

2-color graphics mode (mode 6) can prevent PCjr users from being

able to use certain software.

Other computers that use this mode can change the foreground color,

however the background color is always be black.

The advanced graphic capabilities of the PCjr permit program the

background color to be selectable.  While this feature permits the

PCjr to display superior graphics in PCjr-CGA and Tandy (TGA)

modes, a compatibility problem which concerns BIOS can cause

programs designed to run on other computers to operate incorrectly.

If a program selects mode 6, then issues the command that changes

background color on a PCjr, the foreground color will change on

other computer.  When the same program is run on a PCjr, the

background color changes, as it should.

As a result, screens may become difficult, or impossible, to read.

The Patton vs. Rommel program (Electronic Arts) is a good example.

When Patton vs. Rommel is run on other computers the program issues

a command to set the screen color to white.  This results in white

text on a black background (remember: the background is always

black in this mode on other computers).

When the same program is run on a PCjr the program gets what it

asks for, a white background and foreground.  The result is a blank

white screen.

Other incompatibilities exist in the PCjr's INT 10 routine that

cause more subtle problems.  Mode 5 (320 x 200 4-color B&W) palette

colors are different on the PCjr and modes 4 and 5 can have the

color intensity changed on the PC, but not on the PCjr.

INT10FIX makes no attempt to correct these subtle BIOS

incompatibilities, although it does prevent the bug which prevents

Patton vs. Rommel from running from causing problems on a PCjr.

INT10FIX also corrects the same problem which shows up with the

Hires-mode Procedures in Turbo Pascal.  As a result, INT10FIX will

also correct the display problems with programs compiled with Turbo

Pascal.

If you are using a PCjr, and you do not have a Compatibility

Cartridge installed, it is recommended that you running INT10FIX

every time you start your computer.

If you have a PCE Compatibility Cartridge installed you do not need

(and should not use) INT10FIX.  The Compatibility Cartridge makes

the PCjr's INT 10 routine 100% compatible with other computers,

thereby preventing abnormal operation when INT10 is used.

For more information on the PCE Compatibility Cartridge contact:

PC Enterprises

"The jrProducts Group"

Customer Service: 1-908-280-0025

Order and Catalog Hotline: 1-800-922-PCjr

The PCjr Lives

There is a Reason!