INT48FIX DOCUMENTATION

Published by:

PC Enterprises

2400 Belmar Blvd,  Wall NJ  07719

Author: P. Joseph Calabria

INT48FIX is a PCjr utility that solves several keyboard problems

which can exist on a PCjr - even after the PCjr is upgraded to

include a full size keyboard.

When INT48FIX is executed, the PCjr's usual INT 48 routine is

replaced.  The new routine that provides greater PC compatibility.

To understand the problem solved by INT48FIX you must first know

that most other computers come with keyboards which have at least

83 keys while the PCjr came with a 62-key keyboard.  Even though

the PCjr's keyboard has fewer keys, the PCjr can still generate all

the key codes which can be produced on an 83-key keyboard.  To do

this, a special BIOS routine is run whenever you press a key.  This

routine is called INT 48.

INT 48's job is to evaluate each key that you press.  If the key

that you press is a normal key, the letter A for example, INT48

simply passes the key code to the PCjr's usual keyboard handling

routine.

However, when a key code which has a special meaning is received,

INT 48 converts the key code to the code that a full size keyboard

would normally send.  For example, keep in mind that the PCjr's 62-

key keyboard does not have a BACKSLASH key (\).  In order to

generate the backslash character you must first press the ALT key,

then press the SLASH key.

When INT 48 receives this combination of keys, it always converts

the SLASH key to a BACKSLASH.  In this situation all is fine if you

wanted to input the backslash character, however if you actually

wanted to send an ALT key, then a SLASH key the problem occurs.

As a result of this translation, a PC program can never receive the

ALT-SLASH combination.  That's why programs such as Sidekick, which

require pressing the ALT-SLASH combination will not work correctly

on a PCjr.

When you have a full size keyboard, your keyboard can send the same

key codes the computer as any other full size keyboard.  As a

result, the key code translations performed by INT 48 can become an

obstacle.

The INT48FIX routine strips the usual INT 48 routine of key code

translation routines which are no longer required when a full size

keyboard is used.

We recommend all PCjr users who use a full size keyboard install

and use INT48FIX each time DOS is started.  PCjr users who use

either the chicklet or replacement style PCjr keyboard must NOT use

INT48FIX as doing so removes support for keyboard translations

which are needed when you use on the 62-key keyboards.

PCjr owners who use the PCE Compatibility Cartridge or Game-Master

sidecar, do not need and should not use INT48FIX.

For more information on the PCE Compatibility Cartridge contact:

PC Enterprises

"The jrProducts Group"

Customer Service: 1-908-280-0025

Order and Catalog Hotline: 1-800-922-PCjr

The PCjr Lives.

There is a Reason!