RELEASE NOTES

S3 Flat Memory Model Windows 3.1 Driver 2.2 Release Notes

Release Date:	November 5, 1993

1.	The Flat Memory Model driver does not support Windows 3.1 Standard Mode.

2.	New features and optimizations:

·	Support for the following 805i modes:  640x480x32bpp and 800x600x32bpp.

·	Text optimizations - Font-Caching is enabled automatically on VRAM-based products and all ISA bus instances.  Non-font caching is used for 8bpp resolutions on the local bus with DRAM configuration.  A profile string ( FONT-CACHING=[ON|OFF] ) in the Display Section of the SYSTEM.INI file can be utilized to enable or disable this function.

·	Source-Copy Function - The read-ahead cache of the 486 processor was utilized to gain a performance increase.

·	Polygon-Support - Up to 400 vertices can now be supported if the profile string in the SYSTEM.INI file is enabled for polygon support.  The default setting is ON which means polygons with 400 vertices are supported.

·	Ellipse and Circle Output - The default value is YES.  Refer to the profile string ELLIPSE-SUPPORT=YES in the SYSTEM.INI file.  Turning off the circle support will inform the GDI of a lack of Circle/Ellipse support during the initialization.  Support consists Diameters between 2 and 2048 pixels and solid or hollow brushes are supported.

3.	This release also consists of bug fixes.

4.	The following resolutions are supported:

Resolutions Supported - Disk 1

80X/805i/928 with 8/16 Bit RAMDACs

Pixel				Memory	Font
	Resolution	Colors		Required	Type

640x480		256		0.5MB		Fixed
	640x480		32768		1MB		Fixed
	640x480		65536		1MB		Fixed
	640x480		16.7M (24bpp)	1M		Fixed	
*	640x480		16.7M (32bpp)	2M		Fixed
	800x600		16		0.5MB		Large/Small
	800x600		256		0.5MB		Large/Small
	800x600		65536		1MB		Large/Small
*800x600		16.7M (32bpp)	2M		Large/Small
	1024x768		16		0.5MB		Large/Small
	1024x768		256		1MB		Large/Small
	1024x768		65536		2MB		Large/Small
+1152x864		256		1MB		Large/Small
	1280x1024	16		1MB		Large/Small
	1280x1024	256		2MB		Large/Small

*805i mode only.

+80X Rev C, 805i and 928 Rev E, G mode only.

Resolutions Supported - Disk 2

928 with BrookTree '485 RAMDAC

Pixel				Memory	Font
	Resolution	Colors		Required	Type

640x480		256		0.5MB		Fixed
	640x480		65536		1MB		Fixed
	640x480		16.7M		**2MB		Fixed
	800x600		16		0.5MB		Large/Small
	800x600		256		0.5MB		Large/Small
	800x600		65536		**2MB		Large/Small
	800x600		16.7M		**2MB		Large/Small
	1024x768		16		0.5MB		Large/Small
	1024x768		256		1MB		Large/Small
	1024x768		65536		4MB		Large/Small
	1024x768		16.7M		4MB		Large/Small
*1152x864		256		1MB		Large/Small
	1280x1024	16		1MB		Large/Small
	1280x1024	256		2MB		Large/Small
*1280x1024	65536		4MB		Large/Small

*928 Rev E mode only
**The 928 Rev D chip requires 4MB of memory for these modes.

Driver Limitations:

1.	Graphics in a DOS Box is not fully supported in 800 x600 x 8bpp with 0.5MB of memory; for example, a distorted screen image is produced when running DOSSHELL in Graphics mode in a DOS windows box.  The limitation occurs because not enough offscreen memory is available in 800 x 600 x 8bpp in 0.5MB.

2.	Clipboard - Horizontal lines appear after saving a print screen image.  This will be fixed in a later release.

3.	MS Japanese Windows - Extra text is displayed under a DOS window.  This occurs after switching from a: to c: dos prompt.  This will be fixed in a later release.

4.	Win 32s Freecell - System hangs after running Freecell under Win 32s.

5.	Notepad - The cursor may jump when transitioning from one cursor to another.

6.	Word for Windows - Problems may occur when copying bitmaps from Paintbrush in 32bpp.

7.	MS Visual C++ - The fill routine in bitmap editor of Microsoft Application Studio displays improper images.

Sample OEMSETUP.INF File

This section lists a sample OEMSETUP.INF file which displays the necessary modifications for setting the Linear Address Aperture Base.  The lines displayed in bold are changes which the OEMs must make.  The value for the Aperture Base must be between 0 and 31 depending on how the hardware has been decoded.

[data]

Version="3.10"

; Names of the disks Setup can prompt for.

[disks]

1 =., "S3 Flat Model Driver Disk",disk1

[display]

;

D608 = 1:s3flat.drv, "S3 2.2F2 640x480 256 colors","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D0608

;

D615 = 1:s3flat15.drv,"S3 2.2F2 640x480 32K colors","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D0615

;

D616 = 1:s3flat.drv, "S3 2.2F2 640x480 64K colors","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D0616

D632L = 1:s3flat.drv, "S3 2.2F2 640x480LC 16.7M colors","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D0632LC

;

DL804 = 1:s3flat.drv, "S3 2.2F2 800x600 16 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D804L

DS804 = 1:s3flat.drv, "S3 2.2F2 800x600 16 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D804S

DL808 = 1:s3flat.drv, "S3 2.2F2 800x600 256 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D808L

DS808 = 1:s3flat.drv, "S3 2.2F2 800x600 256 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D808S

DL816 = 1:s3flat.drv, "S3 2.2F2 800x600 64K colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D816L

DS816 = 1:s3flat.drv, "S3 2.2F2 800x600 64K colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D816S

DL832 = 1:s3flat.drv, "S3 2.2F2 800x600LC 16.7M colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D832LLC

DS832 = 1:s3flat.drv, "S3 2.2F2 800x600LC 16.7M colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D832SLC

;

DL1004 = 1:s3flat.drv, "S3 2.2F2 1024x768 16 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1004L

DS1004 = 1:s3flat.drv, "S3 2.2F2 1024x768 16 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1004S

DL1008 = 1:s3flat.drv, "S3 2.2F2 1024x768 256 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1008L

DS1008 = 1:s3flat.drv, "S3 2.2F2 1024x768 256 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1008S

DL1016 = 1:s3flat.drv, "S3 2.2F2 1024x768 64K colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1016L

DS1016 = 1:s3flat.drv, "S3 2.2F2 1024x768 64K colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1016S

;

DL1108 = 1:s3flat.drv, "S3 2.2F2 1152x864 256 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1108L

DS1108 = 1:s3flat.drv, "S3 2.2F2 1152x864 256 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1108S

;

DL1204 = 1:s3flat.drv, "S3 2.2F2 1280x1024 16 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1204L

DS1204 = 1:s3flat.drv, "S3 2.2F2 1280x1024 16 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1204S

DL1208 = 1:s3flat.drv, "S3 2.2F2 1280x1024 256 colors LF","100,120,120", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1208L

DS1208 = 1:s3flat.drv, "S3 2.2F2 1280x1024 256 colors SF","100,96,96", 1:vgacolor.2gr, 1:vgalogo.lgo, 1:vdds3911.386, 1:s3911grb.3gr,, 1:vgalogo.rle, D1208S

;

S3_24    = 1:S3_24.drv, "S3 2.2F2 640x480 16 Million colors ",              "100,96,96",   1:s3vga.2gr, 1:vgalogo.lgo,   1:vddS3.386,  1:s3vga.3gr,,           1:vgalogo.rle, s3

;

[D0608]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=640"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D0615]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=640"

,,system.ini, DISPLAY,"color-format=",   "color-format=15"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D0616]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=640"

,,system.ini, DISPLAY,"color-format=",   "color-format=16"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D0632LC]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=640"

,,system.ini, DISPLAY,"color-format=",   "color-format=32"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

;

;800 by 600 information profiles:

;

[D804L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D804S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D808L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D808S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D816L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=16"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D816S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=16"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D832LLC]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=32"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D832SLC]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=800"

,,system.ini, DISPLAY,"color-format=",   "color-format=32"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

;

;1024 by 768 information profiles:

;

[D1004L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1004S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D1008L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1008S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=96 "

[D1016L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=16"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1016S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1024"

,,system.ini, DISPLAY,"color-format=",   "color-format=16"

,,system.ini, DISPLAY,"dpi", "dpi=96"

;

;1152 by 864 information profiles:

;

[D1108L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1152"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1108S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1152"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=96"

;

;1280 by 1024 information profiles:

;

[D1204L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1280"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1204S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1280"

,,system.ini, DISPLAY,"color-format=",   "color-format=4"

,,system.ini, DISPLAY,"dpi", "dpi=96"

[D1208L]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1280"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=120"

[D1208S]

,,system.ini, DISPLAY,"aperture-base",   "aperture-base=0"

,,system.ini, DISPLAY,"polygon-support",   "polygon-support=no"

,,system.ini, DISPLAY,"screen-size=",    "screen-size=1280"

,,system.ini, DISPLAY,"color-format=",   "color-format=8"

,,system.ini, DISPLAY,"dpi", "dpi=96"

; copy these fonts depending on the 386 grabber being used

[s3911grb.3gr]

1:CGA40WOA.FON,1:CGA40850.FON

1:CGA80WOA.FON,1:CGA80850.FON

1:EGA40WOA.FON,1:EGA40850.FON

1:EGA80WOA.FON,1:EGA80850.FON

[sysfonts]

1:vgasys.fon,"VGA (640x480) resolution System Font", "100,96,96"

1:8514sys.fon,"8514/a (1024x768) resolution System Font", "100,120,120"

[fixedfonts]

1:vgafix.fon,"VGA (640x480) resolution Fixed System Font", "100,96,96"

1:8514fix.fon,"8514/a (1024x768) resolution Fixed System Font",  "100,120,120"

[oemfonts]

1:vgaoem.fon,"VGA (640x480) resolution Terminal Font (USA/Europe)", "100,96,96",1

1:8514oem.fon,"8514/a (1024x768) resolution Terminal Font (USA/Europe)", "100,120,120",1

[fonts]

1:SSERIFE.FON, "MS Sans Serif 8,10,12,14,18,24 (VGA res)", "100,96,96"

1:SSERIFF.FON, "MS Sans Serif 8,10,12,14,18,24 (8514/a res)", "100,120,120"

1:COURE.FON, "Courier 10,12,15 (VGA res)", "100,96,96"

1:COURF.FON, "Courier 10,12,15 (8514/a res)", "100,120,120"

1:SERIFE.FON, "MS Serif 8,10,12,14,18,24 (VGA res)", "100,96,96"

1:SERIFF.FON, "MS Serif 8,10,12,14,18,24 (8514/a res)", "100,120,120"

1:SYMBOLE.FON, "Symbol 8,10,12,14,18,24 (VGA res)", "100,96,96"

1:SYMBOLF.FON, "Symbol 8,10,12,14,18,24 (8514/a res)", "100,120,120"

1:SMALLE.FON, "Small Fonts (VGA res)", "100,96,96"

1:SMALLF.FON, "Small Fonts (8514/a res)", "100,120,120"

1:ROMAN.FON, "Roman (Plotter)",   "CONTINUOUSSCALING"

1:SCRIPT.FON, "Script (Plotter)", "CONTINUOUSSCALING"

1:MODERN.FON, "Modern (Plotter)", "CONTINUOUSSCALING"

[s3]

,,system.ini,s3_24.drv,"WidthXHeight=","WidthXHeight=640x480"

,,system.ini,s3_24.drv,"FontSize=","FontSize=small"