USING THE BB1004/BB1008/BB2016 WITH BBS SOFTWARE

************************************************

The BB1004/BB1008/BB2016 boards are compatible with many BBS

software packages.  The information provided below should assist

you with setting up the BocaBoard for use with your bulletin board

software.

I/O PORT ADDRESS

****************

The starting I/O port address for the first port of the

BocaBoard is set with jumpers located on the BocaBoard.  I/O

addresses are assigned in blocks of eight 10-bit addresses.

Your software may ask for the starting address and the offset

for additional ports.  Check your software documentation for

supported addresses and insure that the BocaBoard is set for an

address supported by your software.

The offset for additional ports on all BocaBoards is 8.

For example, if you have a BB1008 set at address 0100, during

the BBS software installation, you would enter the following:

BASE I/O ADDRESS:  0100

# OF PORTS:        8

OFFSET:            8

PORT STATUS REGISTER

********************

The BocaBoards are designed so that all of the ports on the

board can share one interrupt request level (IRQ).  For example, if

the IRQ jumper on the BB1008 is set to 5, all 8 ports will share

IRQ 5.  This is accomplished through the use of a Port Status

Register (PSR) which is provided to identify which port(s) on the

board caused an interrupt.  This register appears at the eighth or

last address for each port.  Remember: each port on the board uses

a block of eight addresses.  All ports share the same register with

one bit in the register representing the interrupt status of each

port.

Some software (Major BBS, for example) does not require the

use of an interrupt or a Port Status Register.  If this is the

case,you should remove the IRQ jumper from the BocaBoard.

If your software DOES require an interrupt and a Port Status

register, make sure that the BocaBoard is jumpered for an IRQ

supported by your software.  Then, during installation of the

software, you would input the appropriate information as in the

following example:

You have a BB1008, set to I/O address 0100 and IRQ 5.

Your software prompts:              You respond:

________________________            ______________

Base I/O Address :                   0100

# Of Ports :                         8

Offset:                              8

Shared IRQ:                          5

1st Port PSR Address:                0107

IMPORTANT NOTES

***************

The RJ-12 to DB-25 connectors shipped with the BB1004/BB1008

are pinned-out for terminals (DTE), not modems (DCE).  These

connectors WILL NOT WORK with your modem.  See the wiring diagram

in the BocaBoard Manual for correct wiring.

If you have the BB2016, which uses RJ-45 (10-wire) cable, and

you do not wish to make your own RJ-45 to DB-25 DCE connector, Boca

Research provides the IODCE2, available through your supplier.

This includes two 2-meter RJ-45 (10-wire) cables and two RJ-45 to

DB-25 DCE connectors for use with modems.

As the BB1004/BB1008 use six-wire RJ-12, all of the signals

necessary for use with high-speed modems are not provided.  We

recommend that users of 14.4 Kbps modems use the BB2016, which

provides ten-wire RJ-45 connections.  See the wiring diagram in

the BocaBoard Manual for pin-outs and wiring diagrams.

We recommend that you contact the BBS Software Vendor for

questions about compatibility between the BocaBoards and their

software.  The information contained above concerning I/O

addresses, port offset, and port status registers will provide the

software vendor with the information necessary to determine if the

BocaBoards will work properly with the software.

Boca Research currently runs two Bulletin Board services, one

using Galacticomm (Major BBS) and one using TBBS, both in

conjunction with the BB2016.  Other BBS services have not been

tested.

02/03/94