PCJRIPS v1.0

INSTALLATION AND OPERATION INSTRUCTIONS #80106

Copyright (c) 1993 PC Enterprises 1-908-280-0025

GENERAL

PCJRIPS.COM is a utility that allows a PCjr computer to use PCjr

internal commands even after the PCjr is enhanced to use products

or operating systems that prevent internal commands from being

used.

Internal commands are commands provided on ROM devices found in

PCjr cartridges and certain other expansion products.  Examples

of internal commands include the BASICA command available if you

have Cartridge Basic, the JRSYNC and JRSYNCNP commands found on

the PCE Keyboard Buffer Cartridge, and the JRVINT21 command

provided by the PCE jrVideo Cartridge.

Internal commands simplify the use of the PCjr because the

commands are always available, and therefore never need to be

present on any disk or diskettes that you use.

When the PCjr is operated with any version of IBM DOS from 2.10

through 4.01 the computer can use internal commands

automatically, as long as DOS detects the System Identification

Byte of the PCjr when the computer is started.   Later versions

of IBM DOS, in addition to operating systems provided by other

companies (i.e. Digital Research's DR DOS), do not provide

support for internal commands.

PCJR USERS AFFECTED

PCjr users that install Microsoft or IBM DOS 5.00 or later, or

any other operating system that does not support PCjr internal

commands need to use PCJRIPS in order to use commands built into

cartridges and other expansion products that they own.

In addition, PCjr users that install any product that changes the

computer's identification byte from that of a PCjr to the

identification byte of an IBM PC also need PCJRIPS.

A partial list of products and operating systems known to require

PCJRIPS is indicated below:

PRODUCTS THAT REQUIRE USING PCJRIPS

PC Enterprises Configuration Cartridge

PC Enterprises Configuration Plus Cartridge

PC Enterprises Display-Master VGA Cartridge (rev 2)

Racore PC I. D. Cartridge

OPERATION SYSTEMS THAT REQUIRE USING PCJRIPS

IBM or Microsoft DOS versions 5.00 and later

Digital Research DR-DOS 6.00

DESCRIPTION OF PROBLEM

The ability to use internal commands is a feature of the PCjr

that is not found on most computers.  As a result, many operating

systems do not support this feature.

IBM DOS versions 2.10 through 4.01 support internal commands

when used on a PCjr.  Each time a computer is started with one of

these DOS versions, DOS checks to determine whether it is running

on a PCjr.  If the identification byte of a PCjr is found, DOS

automatically provides support for internal commands; however, if

the identification byte of any other computer is detected,

internal commands are not supported.

PCjr users that install operating systems that do not support

internal commands soon find that they can no longer access

commands found built into their cartridges and other expansion

products.  In addition, PCjr users that install a product that

changes the computer's identification byte from that of a PCjr to

that of a genuine PC also lose the ability to use internal

commands.

SOLUTION

PCJRIPS is a program that allows you to access internal commands

regardless of the circumstances that otherwise prevent you from

using these commands.  To use PCJRIPS simply type PCJRIPS

followed by the internal command that you would like to execute.

Example: A-> PCJRIPS BASICA

PCJRIPS [/? or /L] [internal_command [command_parameters]]

/? displays a help screen for the PCJRIPS program

/L lists all the internal ROM commands currently installed

in your computer

internal_command is the command you wish to execute from ROM

command_parameters are the parameters or options you wish to

pass along to the internal command.

When using the /L parameter of PCJRIPS, nonsence will be

displayed on the screen when certain products are installed.

Please ignore any nonsense displayed, as it will not cause any

problems (Nonsence is displayed when a product uses ROM code

designed to be compatible with other computers.  Examples include

the Cirrus VGA BIOS used with the PC Enterprises' Display-Master

Cartridge and the Seagate or Future Domain BIOS provided with the

PCE SCSI Host Adapter).

The PCJRIPS program does not change or influence the way the

internal ROM command works.  If you need help using the internal

ROM command or its parameters, refer to the documentation that

accompanied the product that provides the internal command.

PCJRIPS is an external DOS command.  This means that the

PCJRIPS.COM file must be on your disk to use the utility.

PROCEDURE

The PCJRIPS program will allow internal commands to run exactly

as they were designed to.  Normally you would enter the command

at the DOS prompt with its parameters.  When using the PCJRIPS

program, simply type the name PCJRIPS in front of the internal

ROM command as it is normally used.  Example:

PCJRIPS BASICA MYPROGRM.BAS

If your computer has internal commands that must be executed each

time the computer is run, you can use PCJRIPS in your

AUTOEXEC.BAT file just as you use it at the DOS command line.

The following is an example of how to create an AUTOEXEC.BAT file

that is commonly used by many PCjr users who have a jrExcel-

lerator and a hard drive system:

COPY CON AUTOEXEC.BAT  [Enter]

PCJRIPS DISKSYNC  [Enter]

PCJRIPS FASTSPD  [Enter]

PROMPT $P$G  [Enter]

^Z  [press the Function 6 key then Enter]

Please remember, to run PCJRIPS from your AUTOEXEC.BAT file, the

PCJRIPS.COM file must be copied to your start up disk.

TESTING

If you would like additional help from PC Enterprises, help is

available.  Computer systems vary widely from user to user, so we

require you have the following information when you call:

All hardware items currently installed on your PCjr

The name and version of your PCjr memory management software

The DOS version and brand you use

PC Enterprises 1-908-280-0025

ADDITIONAL INFORMATION

Permission is granted to distribute PCJRIPS.COM and PCJRIPS.DOC

free of charge provided the two files are distributed together

and no portion of either file and/or the contained messages have

been altered.

PC Enterprises has been dedicated to the support of the IBM PCjr

since 1984.  PCjr owners can receive a FREE PCjr products catalog

by calling 1-800-922-PCJR.